Guitar Hero Pico Controller

MicroPython - Project Notes

# 1. Project Overview

This project takes inspiration from the Guitar hero a late 1990s early 2000s famous arcade game and later sold as a home entertainment system. Basically, what guitar hero is, a guitar(with buttons and a strum) that hooks up to your PC and run a small game where you get notes falling on the screen and you try to hit them on time to get score.

In essence, this project is the perfect embodiment of a full stack (in terms of back end and front end work done) learning experiment as it has a fair share of embedded coding (for the guitar/controller), some front end game dev with interactive I/O and finally some designing element to pack the package as a whole.

## 1.0 Back-end

This documentation is more so be going to be on about the back-end part using Micro Python, under whose jurisdictions include just making the Raspberry Pi Pico work as a USB HID controller for the game. In this project the backend responsibilities not only include the software but also the hardware side of things as by the end of this a working model is to be ready, so this documentation not only entails about the software side of things but also digs into the hardware side to a respected degree.(A NOTE TO KEEP IN MIND, it goes just barely into the hardware side, at max just wiring configuration and some decisions, THIS DOCUMENTION LEAVES NO EXPECTATIONS on the user to code THE SDK or ASSEMBLY BOOT SEQUENCE for the MICROCONTROLLER, through the course of this documentation we will be sticking to the provided respected SDKS for a number of languages we are going to be using)

## USB HID controller

The Raspberry Pi Pico is going to be acting as a USB HID controller for the game. Using Micro Python we cant really make it a HID USB device as Micro Python doesn’t support any HID USB library such as Adafruit for Circuit Python and TinyUSB for C. Though one thing we can do is to have it mimic as a HID device by taking advantage of the allowed communication with the PC over the Serial port. We can use Pyserial the library and read data sent over the port, but one major drawback of this method is that though it might work you have to define all the gamepad features into the Reader_file.py containing the Pyserial library as these come preinstalled in HID Libraries but here we kind of have to make our own.

## 1.2 Input Medium

This USB HID controller mimicry for a Guitar Hero style game using Micro Python will take 6-buttons, upon my choice and by no means are we limited to 6, if the board can spare a GPIO pin we can keep on adding as many buttons as up to our hearts desire.

Five of said buttons act as fret/note buttons (any combination can be held simultaneously), and one buttons acts as the strum or at least the replacement for the swinging motion as is in an actual guitar, hence this strum button MUST be held for any note input to be registered.

- 5 note buttons --- can be pressed in any combination

- 1 strum button --- must be held otherwise all input is disregarded

💡 Why strum as a button? Much like how in an actual guitar we need the swing motion, this version on controller try’s to replace that keeping it cheap and frankly said easier on the code side, hence button press logic used for strum

# 2. Wiring

One of the most sensitive part of working with a microcontroller is to do with the wiring, as just unknowingly connecting random wires can short the whole thing (in actuality its not as easy as this but very much so possible).

Down below the table shows specification on each button with a corresponding GP number from the board, we define the GPs (we can choose any of the available 26 pins on the board) to the buttons now and will keep them referencing using the same GP number as a premise. (NOTE GP Pin number is different than pin number and normally you can find the GPIO numbers either printed underneath the physical board itself or refer to a datasheet ladled diagram). Another of the Pico’s useful quirks is that it has a built-in internal pull-up resistor, making our life easier in this case scenario.

The wiring itself will go as follow: we are going to use Pico’s built in internal pull-up resistor for all the 6 buttons. Wiring as such, one leg on each button is connected to respective GPIO pin and the other to ground pin (GND) on the board. By using the pull-up register all buttons initially will show 1/True state and when pressed they will be grounded and show 0/False. This is to be taken note of in the coding part.

| Button | GPIO Pin | Bitmask |
| --- | --- | --- |
| Note 1 | GPIO 2 | 0x01 (00000001) |
| Note 2 | GPIO 3 | 0x02 (00000010) |
| Note 3 | GPIO 4 | 0x04 (00000100) |
| Note 4 | GPIO 5 | 0x08 (00001000) |
| Note 5 | GPIO 6 | 0x010 (00010000) |
| Strum | GPIO 7 | 0x80 (10000000) |

⚠️ Active LOW Because we use pull-ups, an unpressed button reads 1 and a pressed button reads 0. The code inverts this while building the bitmasks.

# 3. Bitmask Protocol

The bitmask is set so the communication between the Pico and the PC is a single byte and sent over USB serial when using Micro Python (or over HID descriptor report when using Circuit Python). The layout for Circuit Python is:

Bit 7  	(0x80)        Strum button

Bit 4  	(0x10)        Note 5

Bit 3  	(0x08)        Note 4

Bit 2  	(0x04) 	 Note 3

Bit 1  	(0x02)		 Note 2

Bit 0  	(0x01) 	 Note 1

Bits 5 and 6 are unused.

We can change this sequence as it is entirely up to us which bit to allocate to what note. NOTE: as this is only possible because in Micro Python, we are sending data over Serial port (is viewable in Thonny’s shell).

For the resulting byte after an input, buttons are combined with bitwise OR. This allows for multiple buttons to be pressed at the same time. For example, strum + Note 1 + Note 3:

0x80  (strum)

| 0x01  (note 1)

| 0x04  (note 3)

= 0x85  (10000101)

- 0x80 is returned when the strum button is pressed

- If strum is not held, the Pico sends nothing (or 0x00 on change)

- The PC side also safety-checks: if strum bit is 0, all notes are forced too False

# 4. Pico-Side Code

# Using Micro Python Pico code has only one file that can just be flashed once and used until re-flashed or BOOTSEL.

## 4.0 Flashing the Firmware

On a fresh Pico we start off by Flashing the Pico with Micro Python firmware (uf2) from Thonny holding the Bootsel. Then we copy the code file to the Pico as main.py. This allows it to run the code every boot after that unless re-flashed.

## 4.1 Defining the Buttons

We start off by having the correct import as show in the code snippet below:

import board

Button = Pin(2, Pin.IN, Pin.PULL_UP)        # puts GP2 as button

We can do this for each individual button or just use a for loop like such:

import board

Buttons = [ Pin ( i, Pin.IN, Pin.PULL_UP) for i in range(2,8) ]      # makes an array of buttons

assigned to GP 2-7

## 4.2 Debouncing

Bouncing a property of physical buttons, where they don't cleanly transition between states. The metal contacts bounce for ~10ms, producing rapid false reads. The denouncer job is to filters this out using software. This is not the only approach to this solution we can also use hardware fixes, but they just entail using higher calibre equipment making this cost more but even then, there’s still small amount of noise between states that slips through; hence the software solution is preferred over the hardware one. But even with the software solution one dilemma that is often faced is that there are too many methods/algorithms that can be used for this problem but each act as a double-edged sword as it may not be the most optimal or ideal solution for a set piece of code.

In this version I am going to be implementing debouncing logic using time element of the board and use time difference in between states and see if its passes the minimum threshold(configurable now set to 20ms) as we know bouncing occurs ~10ms, so updating at 20ms assures us at least in this use case scenario that all noisy unwanted jumps in the signal are going to be filtered.

Three parallel time lists track every pin:

stable      = [1] * num_pins   # last CONFIRMED state

suspect     = [1] * num_pins   # what we are seeing right now

susp_time   = [0] * num_pins   # when candidate state first appeared

The rule: a new state must hold steady for Debounce_ms (20ms) before it is accepted into stable. Bounce noise flickers too fast to survive this window in the Pico. A real deliberate press will easily surpass 20ms.

raw = pin.value()

if raw != suspect[i]:

# if state changed then resets the timer, don't update stable

suspect[i]   = raw

susp_time[i] = now_time

elif time.ticks_diff(now_time, susp_time[i] >= Debounce_ms):

# if held steady for 20ms then update as stable

stable[i] = raw

Example walking through time:

Time   Raw   suspect    stable   What happenes

0ms     1       1          1      idle

50ms    0       0          1      finger hits, timer started

55ms    1       1          1      bounce! reset timer  (~5ms)

57ms    0       0          1      bounce again! reset timer  (~2ms)

77ms    0       0          0      20ms clean consistent → accepted   (>=20ms)

## 4.3  Building the Bitmask

This part reads the debounced states and constructs the byte to send. Because pins are active LOW (we using pull-up transistor), this logic inverts them:

strum_pressed = (states[5] == 0)   # 0 == not pressed (pull-up)

if not strum_pressed:

return 0x00                    # strum not held returns 0x00 (nothing)

bitmask = Strum_bit                # start with 0x80

for i in range(5):

if states[i] == 0:             # note button pressed

bitmask |= NOTE_BITS[i]    # OR it to result

## Some bitmask examples

0x00  nothing / signal release

0x80  strum only

0x81  strum + note 1

0x85  strum + note 1 + note 3

0x9F  strum + all 5 notes

0xFF  strum + all notes + bits 5&6 (unused, can’t happen)

## 4.4  Main Loop

This part is configured to run at 100Hz (every 10ms) Pico processor is capable of 133Hz. At this stage it decides whether to send based on two independent conditions:

- state_changed - the bitmask is updated

- heartbeat_due - 500ms has passed since the last send

Either condition triggers a send. The heartbeat prevents the PC side getting stuck thinking a button is held if a packet is missed(windows does fall into the illusion sometimes hence this is required).

prev_bitmask = None    # None forces a send on the very first loop

while True:

bitmask = tr_requirement(read_debounced()

states = debounced()

state_changed = (bitmask != prev_bitmask)

heartbeat_due = time.ticks_diff(now, prev_heartbeat) >= 500

if state_changed or heartbeat_due:

sys.stdout.buffer.write(bytes([bitmask]))

prev_bitmask   = bitmask

prev_heartbeat = now

sleep_ms(10)

This piece of code is EURIKA smart especially besides the heartbeat check and pass the sys.stdout part. One of the major problems with Micro Python is that it doesn’t support any HID libraries so it can’t send data to a PC over it, but one smart round about way is to use the system import and use the stdout function to send data over serial, normally this is used to debug and check for error but sending the data bit over this is highly ingenious as it can be used with the serial module/library and implemented into the Pygame side of the game and technically be baked into the thing itself.

💡 bytes([bitmask]) bitmask is just an integer like 133. Wrapping it in bytes([]) converts it to a raw single byte b'\x85' which is what actually travels over the serial wire. It works in over favour cause keeping track of button pressed as a 8-bit string is easier when perceived by the PC.

# 5. PC-Side Code  (guitar_reader.py)

Install dependencies:  pip install Pyserial pygame. We basically just define a gamepad controller library in itself and just use that to read the previously sent data bytes.

## 5.1  Auto-detecting the Pico Port

The Pico has a fixed USB vendor ID assigned by Raspberry Pi. We scan all connected serial ports looking for it:

ports = serial.tools.list_ports.comports()

for port in ports:

if port.vid == 0x2E8A:      # Raspberry Pi vendor ID

return port.device      # e.g. COM3 or /dev/ttyACM0

If not found automatically it prints the available ports and asks you to pick one.

## 5.2  Decoding the Bitmask

Checks with the defined values(NOTE IT IS IMPORTANT FOR THESE TO BE THE SAME AS DEFINED IN THE MAIN.PY FILE). Unpacks the raw byte back into a friendly dictionary:

def decode_bitmask(byte_val):

strum = bool(byte_val & 0x80)

notes = [bool(byte_val & bit) for bit in NOTE_BITS]

if not strum:

notes = [False] * 5    # safety check

return { "raw": byte_val, "strum": strum, "notes": notes }

# 0x85 (10000101) becomes:

# { "strum": True, "notes": [True, False, True, False, False] }

## 5.3  The GuitarController Class

Bundles the serial port, background thread, and current state into one tidy object.

### Why a background thread?

Your game loop runs at maybe 60fps. The serial reader needs to listen constantly without blocking the game. Threading lets both run simultaneously, the reader sits in the background waiting for bytes while the game loop runs freely.

self._thread = threading.Thread(target=self._read_loop, daemon=True)

self._thread.start()

# daemon=True means: kill this thread automatically when

# the main program exits. No manual cleanup needed.

### The Lock

Two threads touching the same variable at the exact same millisecond causes corrupted data. The lock prevents this:

# Background thread (writing):

with self._lock:

self._state = new_state

# Game loop (reading):

with self._lock:

return dict(self._state)

# with self._lock means "I need _state, everyone else wait"

# Both sides use the same lock so they can never overlap.

### Why dict(self._state)?

Returning a copy (not the object itself) means your game loop holds a frozen snapshot. Even if the background thread updates _state a millisecond later, your game's copy is unaffected for the rest of that frame.

## 5.4  Using It in a Game

from guitar_reader import GuitarController

guitar = GuitarController()

guitar.start()

# in your game loop:

state = guitar.get_state()

# state = { "strum": True, "notes": [True, False, True, False, False] }

if state["strum"] and state["notes"][0]:

print("Note 1 strummed!")

guitar.stop()    # on game exit

# 6. References

## Files

- pico_guitar_micropython.py  →  copy to Pico as main.py

- guitar_reader.py  →  sits alongside pygame game part of code

## Links

- https://docs.micropython.org/en/latest/library/sys.html v https://stackoverflow.com/questions/tagged/micropython?tab=Votes

- https://github.com/vxj9800/bareMetalRP2040

- https://pip-assets.raspberrypi.com/categories/814-rp2040/documents/RP-008371-DS-1-rp2040-datasheet.pdf?disposition=inline https://docs.micropython.org/en/latest/pyboard/tutorial/debounce.html

- Low level Tv @youtube

- Making Stuff with Chriss DeHut @youtube

- Nashallery @youtube

- Robocraze @youtube

- Will Thomas @yotube

To be taken into consideration a lot of the stuff/logic used is a mix or reading/watching over what felt like countless sources be it in blogs, official forums, users reviews, or similar projected done documented over the internet. With that said a lot of it was inspired by said youtubers you gave a nice introduction with Pico in off itself which allowed me to further experiments and make it work(hopefully).