Guitar Hero Pico Controller

CircuitPython - Project Notes

# 1. Project Overview

This project takes inspiration from the Guitar hero a late 1990s early 2000s famous arcade game and later sold as a home entertainment system. Basically, what guitar hero is, a guitar (with buttons and a strum) that hooks up to your PC and run a small game where you get notes falling on the screen and you try to hit them on time to get score.

In essence, this project is the perfect embodiment of a full stack (in terms of back end and front end work done) learning experiment as it has a fair share of embedded coding (for the guitar/controller), some front end game dev with interactive I/O and finally some designing element to pack the package as a whole.

## 1.0 Back-end What Changed from MicroPython

In MicroPython the Pico faked being a controller, it sent bytes over USB serial, and a reader script was required to be instilled in pygame so the PC could translate it. CircuitPython removes the need of sending over serial entirely, or at least manually. The Pico declares itself as a real HID gamepad at the USB level using the Adafruit library which comes built in the Pico Circuit Python SDK latest ver(10.x), so the OS and any game see it as an actual controller hence no extra decoder script needed, though besides the main code file we do need an extra file entailing the boot sequence cause we declare this as a HID (more on that later under sec. 2).

|  | MicroPython | CircuitPython |
| --- | --- | --- |
| Transport | USB serial → Pyserial → PC | Native USB HID → PC |
| PC side needed? | Yes … guitar_reader.py | No … OS handles it itself |
| Pygame sees it as | Serial port | Real gamepad / joystick |

## 1.1 Key differences from MicroPython at a glance

Some of the other dissimilarities are:

| Thing | MicroPython | CircuitPython |
| --- | --- | --- |
| GPIO | machine.Pin | digitalio.DigitalInOut |
| Time | utime.ticks_ms() - int ms | time.monotonic() - float s |
| Pin value | Integer 1 / 0 | Boolean True / False |
| Send data | sys.stdout.buffer.write() | gamepad.send_report() |
| PC needs | guitar_reader.py | Nothing |
| Strum bit | 0x80 (bit 7) | 0x20 (bit 5) |

# 2. Files for the code

CircuitPython requires to splits startup into two stages:

- boot.py … runs once at power on, before USB is fully initialised. This is the place where device configured as a HID gamepad device.

- code.py … the main loop, runs after boot.py. By the time this starts the PC already knows what kind of device the Pico is.

⚠️ Important If boot.py is edited Pico must be fully unplugged and re-plugged for the changes to be registered. A soft reset is not enough, for HID initialization the USB handshake already happened and needs to start fresh.

# 3. Button Mapping

In Circuit Python we initialize the total number if buttons and report length in the HID descriptor file. In our case the HID report is 1 byte and for buttons we are going to assign 8 bits though we only has 6 buttons, reason being with just 6 bits as the report the 2 remaining bits in the byte may corrupt the data, hence it is preferred to just use 8 and define the remaining as 0x00 throughout.

The buttons are numbered 1-8 sequentially by the HID so unlike in Micro Python we can’t just leave the bits 5 and 6 empty when bit masking, the mapping goes as follow (bits 0-7):

Bit 0  (0x01)  Note 1   - Button 1

Bit 1  (0x02)  Note 2   - Button 2

Bit 2  (0x04)  Note 3   - Button 3

Bit 3  (0x08)  Note 4   - Button 4

Bit 4  (0x10)  Note 5   - Button 5

Bit 5  (0x20)  Strum    - Button 6

Bit 6  (0x40)  unused   - Button 7

Bit 7  (0x80)  unused   - Button 8

# 4. boot.py - The HID Descriptor

The HID descriptor is the Pico's CV equivalent handed to the PC on connection. It literally tells the OS the exact what type and length data reports it will receive. The PC reads it only once at the start and never again unless unplugged and re-plugged. One thing to take note of is to keep the name of this file as boot.py unless the Pico wont be able to detect it on boot up and might keep throwing error (the repeatedly flashing the built-in led every 3-5 sec)

## 4.1  The Descriptor Bytes

The boot.py file will be provided to us upon booting the Circuit python file on the Pico and it mostly consist a copy of mostly everything we need for the descriptor bytes. In this section of code we get to see the various data exchages are going to be happening behind the between the Pico and the PC. Its best preferred to leave most of them as is and only bother with the Report count, this is the usage min and max . These are the only bits that describe the data type of the 8 (6 +2) buttons we have defined.

GAMEPAD_REPORT_DESCRIPTOR = bytes([

0x05, 0x01,   # Usage Page (Generic Desktop)

0x09, 0x05,   # Usage (Gamepad)

0xA1, 0x01,   # Collection (Application)

0x85, 0x04,   #   Report ID (4)

0x05, 0x09,   #   Usage Page (Buttons)

0x19, 0x01,   #   Usage Minimum (Button 1)

0x29, 0x08,   #   Usage Maximum (Button 8)

0x15, 0x00,   #   Logical Minimum (0)

0x25, 0x01,   #   Logical Maximum (1)

0x95, 0x08,   #   Report Count (8 buttons)

0x75, 0x01,   #   Report Size (1 bit each)

0x81, 0x02,   #   Input (Data, Variable, Absolute)

0xC0          # End Collection

])

Every line is a key-value pair (a dictionary) in the HID binary format defined by the USB standard. This format cannot changed, it MUST follow the spec exactly or the PC will not understand it.

💡 Why 8 buttons not 6? HID reports must align to full bytes. A byte is 8 bits so 6 bits cannot be declared - it must be a multiple of 8. Bits 6 and 7 are declared but always stay 0. This is normal practice in HID devices as they require bits sequentially.

💡 Report ID 4 The report ID is just a name tag for this report, 4 is picked arbitrarily. It MUST match the report_id = 4 used in code.py when sending reports, otherwise the OS won't know which descriptor to match it against. Hence best practise to leave as.

## 4.2  Registering the Device

This is one of the manual parts in the HID descriptor file, so depending on weither we are using this as a Keyboard, a mouse or a Custom Gamepad we get to define this part and since we are using it as a Custom Gamepad we have to define it as is.

gamepad = usb_hid.Device(

report_descriptor=GAMEPAD_REPORT_DESCRIPTOR,

usage_page=0x01,       # Generic Desktop

usage=0x05,            # Gamepad

report_ids=(4,),       # must match descriptor

in_report_lengths=(1,) # 1 byte per report

)

usb_hid.enable((gamepad,))

In the code snippet above usb_hid.Device creates the device object from the descriptor. usb_hid.enable() is what actually activates it over USB, without this line the descriptor is defined but never presented to the PC. This last line of code also pass it ONLY as a gamepad disabling the default keyboard and mouse on the Pico not the user device hence keeping the device(Pico) clean for any unaccounted intervention.

# 5. code.py - The Main Loop

## 5.1  Imports

In Circuit Python the machine library is replaced by the board and digitalio library. The time library functions the same for the most part (besides some minor small differences). And the usb_hid part brings over the hid parts from the library for reports.

import board      # names pin constants e.g. board.GP2

import digitalio  # GPIO - replaces machine.Pin from Micro Python

import usb_hid    # to find and use the gamepad device

import time       # replaces utime/time from Micro Python

## 5.2  Pin Setup

CircuitPython is more verbose compared to MicroPython. For GPIO we have to set each property separately. Down here it is wrapped in a functrion to avoid repeating three lines six times:

def button(pin):

btn = digitalio.DigitalInOut(pin)

btn.direction = digitalio.Direction.INPUT

btn.pull = digitalio.Pull.UP

return btn

# MicroPython equivalent (one line):

# Pin(2, Pin.IN, Pin.PULL_UP)

The bitmasking goes as follow:

Bit 0   (0x01)     - 		Note 1

Bit 1   (0x02)	   - 		Note 2

Bit 2   (0x04) 	   - 		Note 3

Bit 3   (0x08) 	   - 		Note 4

Bit 4   (0x10) 	   - 		Note 5

Bit 5   (0x20)	   - 		Strum Button

Bits 6(0x40) & 7(0x80) are	 unused

## 5.3  Finding the Gamepad Device

In boot.py file we create the gamepad and register it besides that in code.py we need to find it to send reports. We search usb_hid.devices by matching usage and usage_page. The values found set in boot.py:

gamepad = None

for device in usb_hid.devices:

if device.usage == 0x05 and device.usage_page == 0x01:

gamepad = device

break

if gamepad is None:

raise RuntimeError("Gamepad not found - check boot.py")

The RuntimeError is a safety net so that if boot.py did not run correctly we crash immediately with a clear message rather than contemplating about the million other reasons why it could have failed cluelessly.

## 5.4  Time - ticks_ms vs monotonic

The time library is another difference from MicroPython:

|  | MicroPython | CircuitPython |
| --- | --- | --- |
| Function | utime.ticks_ms() | time.monotonic() |
| Returns | Integer milliseconds | Float seconds |
| Overflows? | Yes --- use ticks_diff() | No --- plain subtraction fine |
| Example | ticks_diff(now, then) >= 20 | (now - then) >= 0.02 |

ticks_ms() overflows and wraps back to zero eventually which is why MicroPython needs the special ticks_diff() function whereas monotonic() just keeps counting up as a float so we can subtract directly.

## 5.5  Debounce

Identical logic to MicroPython. Few differences that are are:

- btn.value returns a boolean (True/False) not an integer (1/0)  so stable starts as [True] not [1]

- cand_time stores float seconds (0.0) not integer milliseconds (0)

# MicroPython

stable = [1] * num_pins

utime.ticks_diff(now, cand_time[i]) >= DEBOUNCE_MS   # must use ticks_diff()

# CircuitPython

stable = [True] * num_buttons

(now - cand_time[i]) >= DEBOUNCE_TIME                # directly sub allowed

## 5.6  Building the Bitmask

Once again identical logic to MicroPython's. Only difference is comparing to False instead of 0 because pins return booleans. Active LOW still applies so False means pressed, True means not pressed.

strum_pressed = (states[5] == False)   # False = pressed (active LOW)

if not strum_pressed:

return 0x00

bitmask = strum_bit                     # 0x20 - bit 5

for i in range(5):

if states[i] == False:

report |= note_bits[i]

💡 Strum is now 0x20 not 0x80 In MicroPython the bitmask layout was ours to design freely. In HID the report numbers buttons 1-8 sequentially (bits 0-7), so strum as button 6 sits at bit 5 (0x20) instead of bit 7 (0x80).

## 5.7  Sending the Report

This is the key difference from MicroPython:

# MicroPython - sends raw byte over serial wire

sys.stdout.buffer.write(bytes([bitmask]))

# CircuitPython - sends into HID buffer, USB stack delivers it

gamepad.send_report(bytes([report_byte]), report_id=4)

The bytes([report_byte]) wrapping is identical to turn an integer like 133 into a real byte. But instead of travelling down a serial wire to be read by guitar_reader.py, it goes into the USB HID buffer. The OS picks it up and delivers it to any application listening for gamepad input. No reader script needed. This is one of the key advantages Circuit Python has over Micro Python.

## 5.8  Main Loop

Identical structure to MicroPython. Read buttons, build report, send if changed or heartbeat due, sleep 10ms:

prev_bitmask   = None

prev_heartbeat = time.monotonic()

while True:

now    = time.monotonic()

states = debounced()

bitmask = str_requirement(states)

state_changed = (bitmsak != prev_bitmask)

heartbeat_due = (now - prev_heartbeat) >= HEARTBEAT

if state_changed or heartbeat_due:

send_report(bitmask)

prev_bitmask    = bitmask

prev_heartbeat  = now

time.sleep(POLL_INTERVAL)

# 6. Implementation in Pygame Game

No extra script needed. The Pico shows up as a joystick. pygame reads it like any other gamepad:

import pygame

pygame.init()

pygame.joystick.init()

guitar = pygame.joystick.Joystick(0)

guitar.init()

# in game loop:

pygame.event.pump()

note1 = guitar.get_button(0)   # Note 1

note2 = guitar.get_button(1)   # Note 2

note3 = guitar.get_button(2)   # Note 3

note4 = guitar.get_button(3)   # Note 4

note5 = guitar.get_button(4)   # Note 5

strum = guitar.get_button(5)   # Strum

# Strum logic is handled on the Pico —

# notes will only ever be True when strum is also True

# 7. Quick Reference

## Files

- boot.py               - drop onto CIRCUITPY drive root, handles HID declaration

- code.py              - drop onto CIRCUITPY drive root, main loop

- lib/adafruit_hid/  - library folder inside lib/

## Links

- https://docs.micropython.org/en/latest/library/sys.html v https://stackoverflow.com/questions/tagged/micropython?tab=Votes

- https://learn.adafruit.com/circuitpython-essentials/circuitpython-essentials

- https://circuitpython.org/libraries

- https://github.com/vxj9800/bareMetalRP2040

- https://pip-assets.raspberrypi.com/categories/814-rp2040/documents/RP-008371-DS-1-rp2040-datasheet.pdf?disposition=inline https://docs.micropython.org/en/latest/pyboard/tutorial/debounce.html

- Making Stuff with Chriss DeHut @youtube

- Robocraze @youtube

- Will Thomas @yotube

To be taken into consideration a lot of the stuff/logic used is a mix or reading/watching over what felt like countless sources be it in blogs, official forums, users reviews, or similar projected done documented over the internet. With that said a lot of it was inspired by said youtubers you gave a nice introduction with Pico in off itself which allowed me to further experiments and make it work(hopefully).